These are instructions to give students for tracking Dictyostelium using mTrackJ and Image J. I have modified the instructions and added in hints where students either frequently make mistakes or where we have found “bugs” in the software over the years and how to get around them. It also includes how to make a montage of different time points from the movies and the class-specific instructions at the end for what the students need to turn in for their assignment.

Kate Cooper, Loras College

Movie Analysis:

Use ImageJ and mTrackJ plugin make measurements of the distance and speed of cells, directionality, and make a montage of images showing the movement over time for your lab report.  A full lab report will be due from this data.

Downloading Image J and Calibrating your Movie Files:

- Go to this webpage:  http://www.imagescience.org/meijering/software/mtrackj/ and download the MTrackJ_.jar and the Imagescience.jar.  (If you try to just double-click and open these files after they are downloaded, your computer will tell you they are corrupted. Just as long as they are saved somewhere on your computer that you can find them, it’s fine.)

- Open ImageJ and go to the tab “Plugins” then choose “Install.”

- It will open up a window for you to choose the file to install. Go to your “Downloads” folder or wherever you put the jar files and choose the MTrackJ one. Then save it in the “jars” folder when it prompts you to do that.

- Then do the same for the ImageScience jar file.

- Save your .tif movie files somewhere on your computer.

- From within Image J, go to “File” > “Import” > “Tiff virtual stack”  -- THIS IS IMPORTANT!! If you open the movies any other way they will not work correctly.

- Then find your first movie file and click “Open”

- Repeat for your second movie file.

- Also from within Image J, go to “File” > “Open”

- And find and select “Calibration pic2.tif” to open it.

- First you will calibrate your movies for distance.

- Use the Line Selection tool to draw a selection line a known length on the calibration picture (hold down shift to keep it straight). Here we've drawn a 100µm line:

- With the selection line still present on the image, select "Set Scale" from the Analyze menu.

- The “Distance in Pixels” box will be filled in with how long that line was in pixels.

- Fill in the “Known Distance” box with how long you know your line was. (Note what is 100 uM in my picture above.)

- Fill in the “Unit of length” (using ‘um’ for micrometers/microns works, you don’t need the special symbol.)

- “Pixel aspect ratio” just stays at 1.

- *Make sure the ‘global’ box is checked and then this scale will be applied to any file that you have open in ImageJ.*

- Go to “Image” > “Properties” and set frame interval to 60 sec and check the “Global” box and then click “OK.”

Tracking Cells using MTrackJ

** Important!! How are you going to choose what cells to track?  How many are you going to track?  (Hint – you need to be unbiased in your picking of cells, you also need to track at least 10 cells per movie in order to have reliable results, more would be even better. Also, the cells closest to the chemoattractant source are probably experiencing the chemoattractant gradient the best without other cells getting in the way.)**

- Open the MTrackJ program by going to the “Plugins” menu, then “jars” at the bottom, and then “MTrackJ”

- It will open a little menu that looks like this:

- From this menu, click “Add”.  Pick a cell to follow in your first movie and click on it (perhaps in the center of the cell?)  The program puts a dot where you clicked, gives the cell a number, and advances to the next frame.  Click again and repeat until you reach the end of the movie.  When you are done with that cell “Escape” to end that track.  You can then take the movie back to the beginning with the slider at the bottom and start clicking on a new cell.

- COMPUTER PROGRAM BUG:  Sometimes when you click on your first cell, the movie doesn’t advance to the next frame and no number appears, it just still looks like a plus sign. If this happens, just click the red x close button in the upper right corner of the MTrackJ menu, and then reopen the program (ie – start over at step 20)

- Sometimes it might be difficult to see where the cell is under the circles you’ve already clicked. It is sometimes useful to zoom in using the magnifying glass tool in the main ImageJ menu. You can do this in the middle of tracking a cell or in between cells. Click on the  button to go back to tracking the cell.

- Be careful NOT to click ‘Clear’ because it will erase all your tracks!! If you made a mistake in your click, just back up to the previous frame and click in the correct spot. Delete can also be carefully used to delete back a few clicks.

- When you’re done tracking cells for each movie, make sure you click “Save” to save those tracks as a separate file that you can reopen later along with your movie if you need to do more analysis.

- When done, click on Measure (in the MTrackJ menu) and if it gives you the option , check the first two boxes (Display point measurements, Display Track Measurements), but it probably won’t give you the option.  You will get two new windows open that show the analysis of the data. The track measurements is for each entire track.  The point measurements is the timepoint by timepoint analysis of each track.

- This data can be copied and pasted into Excel or saved and the file can be opened as an excel file. (Retype headings as you need them.)

- CHECK TO MAKE SURE ALL THE DATA IS THERE AND THERE AREN’T A BUNCH OF NAs before you go on and then don’t have the data you need later!!! Talk to your professor if there is a problem or a question. (NAs in some columns are okay, but not the column of information you need!)

- Repeat for the other movie.

Analysis of data:

- The columns of data (described in manual) are:

ID: The ID number of the track to which the point belongs.

PID: The ID number of the point.

x [unit]: The calibrated x coordinate of the point. The pixel width and unit of length used here can be set as described above.

y [unit]: The calibrated y coordinate of the point. The pixel height and unit of length used here can be set as described above.

z [unit]: The calibrated z coordinate of the point. The voxel depth and unit of length used here can be set as described above. This quantity is not displayed in the case of 2D image sequences.

t [unit]: The calibrated t coordinate of the point. The frame interval and unit of time used here can be set as described above.

Len [unit]: The length of the track from the start (first) point of the track to the current point (inclusive). Thus the value of this quantity for the end (last) point of a track equals the total length of the track. The pixel calibration and unit of length used here can be set as described above.

D2S [unit]: The distance from the start (first) point of the track to the current point. Thus the value of this quantity is always less than (or at most equal to) the value of Len. The pixel calibration and unit of length used here can be set as described above.

D2P [unit]: The distance from the current to the previous point of the track. The pixel size and unit of length can be set as described above.

v [unit]: The velocity, at the current point, of the object represented by the track. This quantity is computed as the distance from the previous to the current point, divided by the frame interval. The pixel calibration, unit of length, frame interval, and unit of time can be set as described above.

- The mean cell speed for each cell in the MTrackJ: Tracks data file has already been determined for you.

- Enter this data into SPSS/PASW or Excel and graph the average cell speed for all the cells in each of your two treatment groups.

- You can create a box plot or bar graph with SEM error bars in SPSS or a bar graph with SEM error bars using Excel. (What ELSE might you want to do with this data in order to help you answer your hypothesis in the lab?)

- Additionally, you will want to determine Directional Persistence.  We are going to use a simple measure of directionality.  First, figure out the total length of the path the cell followed for those 10 minutes. This can be seen in the MTrackJ: Points data set.  Find the last point for each cell in the Len (uM) column – this is how many uM that cell traveled over those 10 minutes.

- Now, if the cell went in exactly the correct direction that whole time, they would have moved that whole distance in a vertical line toward the chemoattractant source (“up or down” depending on which side you were filming, if you’re looking at the movie). If they spent some of their distance going sideways or away from the chemoattractant, then there won’t be as much displacement in that “Y” direction toward the chemoattractant.

- So, we can figure out the total displacement in the Y direction (up and down in our movie) over 600 sec and then determine a ‘Directional Persistence’ measurement by taking the Y displacement value and dividing by the total distance traveled (Len).

- Find the Y at the first time point.  (Say mine was 321.93) Then subtract the Y for the last time point (265.79) = 56.141 uM is the amount cell moved in the Y direction. (Note: be careful of negatives, only use them when the cell had a total displacement in the opposite direction, not just because the cells were supposed to be going ‘down’ on the screen instead of up. So in one of your movies, you’ll want to change the values to positive ones instead of negative if the cells are going down. Ask me if this is confusing, since I’m sure it is.

- Then, take that Y displacement you just figured out and divide by the total distance traveled (Len). You should get some value between 1 and -1 for each cell.

- Determine the average ‘Directional Persistence’ for cells in each of your two movies and plot these averages in an appropriate graph with appropriate error bars. (again, what else might you want to do with your sets of numbers of directional persistence??)

- Finally, you will want to create a montage of still pictures taken from your movies. To do this first you need to create substacks from your two movies of just the three pictures you want to create a montage with.

- Go to “Image”> “Stacks”> “Tools”> Make Substacks.

- Enter in the slice numbers 1,6,11 – these correspond to 0 min, 5 min, 10 min.

- Save that new stack with the concentration in the name.

- Repeat for the other movie.

- Then go to “Image”>”Stacks”> Tools > Combine.

- Choose each substack and check “Combine vertically.”

- Then it should show you your two substacks, one stacked on top of the other.

- Then go to “Image” > Stacks > Make Montage.

- Make sure it says “3 columns” but only 1 row! Everything else can be left as default.

- This will create a new image file called Montage.

- You should go to “Image” > Type, and then check 8-bit instead of 16-bit.

- SAVE with a new name.

- You can then insert this image file into some other image editing software (or perhaps Word if you are careful with your labels) to put labels along the top and sides. (what is most important to label?)

For your lab write-up:  Full lab write-up. Cite sources in the style of Molecular Biology of the Cell (http://www.molbiolcell.org/site/misc/ifora.xhtml - this website also might be useful for what to put in sections of the paper, though you do not have to strive for the word counts listed, except for the abstract word limit.)

Due date:  ________

Use the new rubric posted to eLearn to help you put together a good lab report/paper! (slightly different than the last paper.)

Title of your paper

Abstract - State the problem, summarize the key findings, and state interpretations and conclusions in 200 words or less.

Introduction – need to cite journal articles as you start broad and give background on this experiment and then narrow in to your question/hypothesis that you are going to be testing in the paper.

Methods section – past tense, don’t write it like directions, ask me if you need more information since I did some things for you.

Results section -

3 figures (Image montage, 2 box/bar graphs – speed, and directional persistence) with figure legends

Text describing results, referring to figures (in order) as you describe the observations, data, and statistics.

Discussion section – interpreting the results, putting them in context with what other researchers have found (citing articles)

Works Cited section